Steps to initialize a YDN station

- Flash OS to 64 GB SD card

  - OS: Raspberry Pi OS Lite (32-bit) Trixie with no desktop environment

  - Use name: ydn and password: ydn

- Connect to Skyroam or other internet router for testing

- Install system packages

  - Sudo apt update

  - Sudo apt install python3-pip

  - sudo apt install python3-yaml

- Connect to Github

  - mkdir ydn

  - sudo apt install git

  - git clone https://github.com/edn-cpu/YDN.git

- Set the station

  - Station/configs/config.yaml

    - Set node_id to # this station will be in chain

- Enable I2C and UART

  - sudo raspi-config nonint do_i2c 0

  - sudo raspi-config nonint do_serial 0

- Install Python packages

  - pip3 install -r requirements.txt

General tasks:

- Upgrade clock_manager to set time

- Add battery cable

- Wire

- RTC

- opc10, opc20, opc40, wind_speed max

- Min batt